PARAMETR-module documentation; parsing command line parameters

Last updated 14.1.97 (PF)

GENERAL

PARAMETR module contains almost all that is needed for parsing

command line parameters. Its implementation may be little bit

dirty and lots of C tricks are used, but the usage of the module

is very easy once you know how to use it! The module consists of

the following files:

PARAMETR.DOC

This file.

PARAMETR.H

Interface for the module. This must be included in the

program using the module.

PARAMETR.C

Implementation of the module. You do not necessarily need to

know the details. Only the understanding how to use it via

Fact file and the .H file is important.

PARAMETR.FAC

Fact file in ASCII format. The user defines here what kind of

command line parameters the program uses. This is the ONLY file

that is allowed to edit by the user.

FORMAT OF COMMAND LINE

The module supports command lines parameters with the following form:

PROGNAME -O -Q2 name1 name2 -X15 -name3 -S5,0,13 -N42

The first one is the name of the program. Other strings

(separated by blanks) are either:

1. Names

2. Options

The options can be given in any order in the command line. Only the

names are assumed to appear in predefined order in respect to each

other. The names are any strings starting with any other character

but the option symbol ('-'). They are considered here as file names,

but can be used in any way the user likes.

Three types of options appear:

1. Boolean options (-O)

2. Numerical options (-Q2)

3. Mulitple numerical options (-S5,0,13)

Each of them are handdled as variables of the type INTEGER.

The combined option allows to parse several parameter values

from the same option symbol. For example -S5,0,13 implies three

different parameters, whose values are input here as 5, 0 and 13.

Boolean options can have only 0/1 value. If the option for any

defined parameter is not found in the command line, predefined

default value will be used.

USAGE OF THE OPTIONS

Once the command line is parsed, each parameter is assigned by

a system variable of any name the user defines. Its value is

then available in the program via the macro:

Value(OptionName)

FACT FILE

A fact must be defined in the fact file for every parameter used in

the program. A fact can look like this:

Fact( ModellingType,

"Modelling type",

'M',    1,  ENUM,   0,    0,   2,      1,      NO,

STATIC, SEMIADAPTIVE, DYNAMIC, mt4, mt5, mt6, mt7, mt8, mt9, mt10,

"Static", "Semi-adaptive", "Dynamic", 0, 0, 0, 0, 0, 0, 0 )

The value of this fact is given in the command line, for example:

-M5

In the software the user can now check its value via the macro:

Value(ModellingType)

The fiels in the facts are:

OptionName

The name for the parameter. In the example this is ModellingType.

The OptionName cannot be used directly, but the macros (explained

later) must be used.

Document

String description of the parameter. In the example "Modelling type".

This string is printed in the help text, etc.

Key (*)

Option symbol from which the parameter value is obtained (in the

example 'M').

Number (*)

Option subparameter (in the example 1).

The fields KEY and NUMBER together identifies uniquely the

parameter. The NUMBER defines the order in which the parameter

is parses after the KEY in case of multiple numerical option.

For eaxmple, in the example combination KEY='M', NUMBER=1

means that the value must be input as -Mx. NUMBER=2 would

imply -Mx1,x2  where different facts must be defined for x1

and x2.

If you do not need the multiple option possibility, then use

always 1 for NUMBER!

In case of BOOLEAN symbols where no numbers are needed,

use 0 for NUMBER! The logic is the following. If the

parameter is not given, its default value is used (must

be set to FALSE). If the parameter appear in the command

line (-M), then its value will be TRUE.

Type

Defines the type of the parameter (ENUM in the example). The

following types are supported:

INT  = Integer.

ENUM = Enumerative type (integer in practice)

BOOL = Boolean type with values: (1=YES, 0=NO).

If the type is set to ENUM, the used enumerative values are

set to those give in the ENUM OF VALUES fields, and their

string representations in the NAME OF VALUES fields. The

upper limit for ENUM values is set to 10.

Value

This is the field in the C data structure where the value is

stored. This field is not needed in the fact file, but cannot

be avoided. So but here 0 value always. The value does not

matter since the value is set automatically in the parsing.

If the corresponding option is not given, the predifined

default value is used.

Min

The smallest allowed value for the parameter. 0 in the example.

Must be defined for ENUM and BOOLEAN types also!

Max

The greatest allowed value for the parameter. 2 in the example.

Must be defined for ENUM and BOOLEAN types also! MIN and MAX

together defines the range of the parameter. It is important to

set these values correctly, for ENUM types also! For BOOLEAN

option, these are not used, but you can define the as Min=0,

Max=1 for the sake of clarity.

Default

Default value is given here. Make sure it is within the range

Min..Max.

SaveToFile

This parameter is actualy not used in the module, but its usage

its up to the program. If you have no idea how to use it, set it

to NO, or zero. Originally it was meant to use in compression

programs to indicate which parameter values should be stored

in the compressed file so that decoder can get them there and

the user does not need to no these when using the compression

program. But as said, depends on the program how this is used.

Enum of values

Names of ENUM values. All 10 names must be uniquely defined,

even in the case of INT and BOOL types. In the example they

are STATIC, SEMIADAPTIVE, and DYNAMIC. For example, in the

program these can be utilized:

if( Value(ModellingType)==DYNAMIC ) ...

The primary purpose of these names is to make the program

more readable and thus support self-documentary coding style.

Note that unique dummy names must be created for those values

names that are not used. In the of INT and BOOL types all 10

names are dummy.

Name of values

String representation for the above ENUM values. In the example

they are "Static", "Semi-adaptive", "Dynamic". The names are

used in the help text, etc.

SUPPORTED ROUTINES

PARAMETR.H file includes all you need including general purpose

routines for help text, macros to access the fact fields, etc.

The following macros are found. In most cases you only need the

Value-macro and nothing else!! The rest are rarely used, for

example, when you want to print better help text than the

automatic PrintOption routine supports.

Macro:		 Type:	  Function:

--------         -------  ---------

Document(x)	 char*	  Returns pointer to the string giving the

string representation of the parametr x.

Key(x)		 char	  Returns the option character where the

parameter x was parsed.

Number(x)	 int	  Returns the order number of the parameter

in respect to other parameters parsed from

the same option.

Type(x) 	 int	  Returns the parameter type as INTEGER.

Value(x)	 int	  Returns the value of the parameter.

SetValue(x,v)	 void	  Set the value v for parameter x.

Max(x)		 int	  Returns the upper limit for the parameter.

Min(x)		 int	  Returns the lower limit for the parameter.

Default(x)	 int	  Returns the default value of the parameter.

SaveToFile(x)	 int	  Return boolean value whether the parameter

should be save into the compressed file.

NameOfValue(x,v) char*	  Return pointer to a string representing

the enum value v for parametr x.

Beside these macros, the following routines are recommended to

be used in the program:

enum __ParameterCoding

{ Omit, FixedBits, FELICS, mt4, mt5, mt6, mt7, mt8, mt9, mt10 };

Routine:		  Function:

--------		  ---------

ParseParameters()	  This routine performs the command line

parsing. Option values are automatically

set, errorneous values rejected, file

names parsed and returned.

PrintOptions()		  Prints the supported options. To be used

as a part of the help text.

PrintSelectedOptions(..)  Prints the chosen parameter values.

To be used after command line is

parsed but before the processing starts.

ParseOption(char* s)	  This routine is needed ONLY if you are

not satisfied with the ParseParameters

routine but want to implement you own.

By this way you can handdle the filen

names etc., but let the module take

care the options.

SetDefaultValuesForParameters()

This routine is already called from the

ParseParameters routine, so you do not

needed. Only if you implement your own

routine, you might need this.

InitializePrintingConditions()

--""--

ReadOptions(f)	 FILE* f  Reads option values from compressed file.

To be used together with the SaveToFile

field.

WriteOptions(f)  FILE* f  Save the selected parameters (for which

the SaveToFile field is set to 1) to the

compressed file.

USAGE OF THE PARSEPARAMETERS ROUTINE

1) Define ParameterInfo-type variable and initialize it as follows:

ParameterInfo paraminfo[N] = { { Name1, "ext1", p1, FT1 },

{ Name2, "ext2", p2, FT2 },

(...)

{ NameN, "extN", pN, FTN } };

This program recognizes N parameters (file names) which are all

stored into strings Name1, Name2, etc. If no file extension is

given in the parameter, the name is catenated with the extension

"ext". No extension is catenated if it is defined as "".

The third field is a "priority number": 0 means that this file

name must always be given. The other files are numbered uniquely

so that the first used parameter is prioritized as 1, the second

as 2, and so on.

The last field defines the type of the file:

INFILE	     Input file

OUTFILE      Output file

INOUTFILE    Input/outputfile

DONTCARE     The usage of this file does not matter.

On the basis of these information, ParseParameters checks that

same names have not been given to the files, except if:

a) the files are both input files

b) the other file is of the DONTCARE type.

Example 1:

ParameterInfo paraminfo[3] =  { { TRFileName, "ts", 0, INFILE },

{ InFileName, "cb", 1, INFILE },

{ OutFileName,"cb", 0, OUTFILE} };

The program can be used either with two parameters, which are

TRFileName and OutFileName, or with three parameters which are

TRFileName, InFileName and OutFileName (in this order). The

optional file name is here given in the middle of the obligatory

file names.

Example 2:

ParameterInfo paraminfo[4] =  { { TRFileName,   "ts", 0, INFILE },

{ InFileName,   "cb", 1, INFILE },

{ OutFileName,  "cb", 0, OUTFILE},

{ DebugFileName, "" , 2, OUTFILE} };

The difference between the previous one is that the user may give

addition file DebugFileName. No explicit extension can be give

by default.

Example 3:

ParameterInfo paraminfo[4] =  { { TRFileName,   "ts", 0, INFILE },

{ InFileName,   "cb", 2, INFILE },

{ OutFileName,  "cb", 0, OUTFILE},

{ DebugFileName, "" , 1 ,OUTFILE} };

This, on the other hand, differs from the previous when the user

gives three parameters. They are now: TRFileName, OutFileName,

DebugFileName, whereas in example 2 the interpretation was the

same as in example 1.

2) ParseParameters routine is called like this:

ParseParameters(argc, argv, N, paraminfo);

where N is the number of parameters. argc ja argv are the

parameters of main().

Notifications:

--------------

When user gives too few or too much parameters, an error message

is shown and the info text of the program is printed by calling

PrintInfo(), which is expected to appear in the main module.

The Parametr module includes the file module.

Features often used but not supported by parametr module:

---------------------------------------------------------

1. If file name is given, it is taken from the previous file name.

This is typical for processing and compression software for

which identical file names are possible because the file types

(and extenstion!) are different. This kind of situations must

be solve in the main module. Do NOT set the priority to 0.

2. If file name is given, a default file name is used. This is

typical for VQ software using default codebook, softwares

outputting additional informative files. This situation must

be solved similarly than the previous one - in main module.

3. Input/output file type can be different. Typical for image

processing software applicable to all types of PBM/PGM/PPM.

The file type (and extension) are explicitely determined

in the main module by DetermineFileType routine, or similar.